Electrically adjustable sports seats Removing and installing motors and senders for height/longitudinal adjustment - ● Front and rear seat height adjustment mechanism moved to highest position. - ● Seat removed Notes: - ◆ Moving seat mechanically forwards and backwards to remove in the event of defective longitudinal adjustment mechanism => Page 72-52. - ◆ After removing seat raising can be achieved by connecting height adjustment motors directly to a voltage source. Connector to motor must be detached. - ◆ Voltage must not be applied to control unit for seat adjustment with memory. - ◆ If seat cannot be raised due to defective height adjustment motor, cut through appropriate drive shaft with cut-off grinder (remove cover seat when doing so) and remove motor. Replace drive shaft that has been cut. Removing

| - ‒ → Unscrew the 4 bolts -arrows- for seat bracket. - ‒ Detach bracket to seat frame. |  |
| --- | --- |

| - ‒ → Screw out nuts -1- and -2- at motors for front/rear height adjustment. - ‒ Cut open cable tie. - ‒ Detach connectors. - ‒ Use plastic-headed hammer to drive runner outwards -arrows- on height adjustment motor side. - ‒ Pull motor for front/rear height adjustment off drive shaft. - ‒ Press sender for front/rear height adjustment out of clamp at motor. - ‒ Detach motor for longitudinal adjustment. - ‒ Detach connectors. - ‒ Detach drive shaft and pull motor for longitudinal adjustment off drive shaft. - ‒ Press sender for longitudinal adjustment out of clamp at motor. Installing Note: Ensure that the two runners are parallel. |  |
| --- | --- |

| - ‒ Refer to installation position (lug) when fitting new sender. - ‒ Attach motor with drive shaft for longitudinal adjustment at runner. - ‒ → Measure fore-and-aft setting of runner on motor side and adjust opposite runner to same dimension. |  |
| --- | --- |

| - ‒ Attach motor for front and rear height adjustment to drive shafts. - ‒ Position both runners on seat frame and insert guide lugs into holes at seat frame. - ‒ → Measure height adjustment setting at front and rear of one runner and adjust opposite runner to same dimensions. |  |
| --- | --- |

| - ‒ → Attach runner to drive shafts -1- in this position and at the same time insert motor securing pins -2-. - ‒ Measure parallel adjustment again. - ‒ Screw runner to seat frame to 20 Nm. - ‒ Screw on motor for longitudinal adjustment to 10 Nm. - ‒ Screw on (6 Nm) height adjustment motors. Note: Synchronisation of sender for longitudinal adjustment/height adjustment senders involves completely traversing entire fore-and-aft, front height and rear height adjustment range. |  |
| --- | --- |