Gearbox identification

Code letters, mechanical unit assignment, transmission ratios, capacities

| Manual gearbox |  | 6-speed 01E, four-wheel drive |  |  |
| --- | --- | --- | --- | --- |
| Code letters |  | DQT | DSY | EDU |
| Production | from | 07.97 | 01.98 | 01.98 |
|  | to |  |  |  |
| Allocation | Model | Audi A6 1998 > | Audi A6 1998 > | Audi A6 1998 > |
|  | Engine | 2.5 l - 110 kW - TDI | 2.7 l - 184 kW- Biturbo | 2.7 l - 169 kW- Biturbo |
| Ratio | Final drive | 37 : 9 = 4.111 | 37 : 9 = 4.111 | 37 : 9 = 4.111 |
| Z2 :Z1=i | 1st gear | 28 : 8 = 3.500 | 28 : 8 = 3.500 | 28 : 8 = 3.500 |
|  | 2nd gear | 34 : 18 = 1.889 | 34 : 18 = 1.889 | 34 : 18 = 1.889 |
|  | 3rd gear | 32 : 26 = 1.231 | 32 : 26 = 1.231 | 32 : 26 = 1.231 |
|  | 4th gear | 27 : 31 = 0.871 | 29 : 30 = 0.967 | 29 : 30 = 0.967 |
|  | 5th gear | 26 : 39 = 0.667 | 29 : 36 = 0.806 | 29 : 36 = 0.806 |
|  | 6th gear | 23 : 41 = 0.561 | 26 : 38 = 0.684 | 26 : 38 = 0.684 |
|  | Reverse gear | 38 : 11 = 3.455 | 38 : 11 = 3.455 | 38 : 11 = 3.455 |

| Code letters |  | DQT | DSY | EDU |
| --- | --- | --- | --- | --- |
| Speedometer |  | electronic |  |  |
| Capacity |  | 2.5 litres 1) | 3.4 litres 1)2) | 2.5 litres 1) |
| Specification |  | Gear oil G 052 911 A SAE 75 W 90 (synthetic oil) |  |  |
| Clutch mechanism |  | hydraulic |  |  |
| Clutch plate ø |  | 240 mm3) |  |  |
| Drive shaft flange ø |  | 130 mm4) | 108 mm | 108 mm |
| Overall ratio iov. in top gear |  | 2.306 | 2.813 | 2.813 |
| Assignment of rear final drive Code letters |  | CUB | DSA EUU | DSA EUU |

1)    Oil capacity was reduced

2)    Oil capacity includes 0.3 l for gearbox oil cooler and oil pipe. When replacing gearbox, oil cooler and oil pipes have to be completely drained

3)    with dual mass flywheel

4)    for tripod joint drive shafts with 130 mm centring diameter

| Manual gearbox |  | 6-speed 01E, four-wheel drive |  |  |
| --- | --- | --- | --- | --- |
| Code letters |  | EEY | EHS | EHT |
| Production | from | 06.99 | 10.99 | 10.99 |
|  | to |  |  |  |
| Allocation | Model | Audi A6 1998 > | Audi A6 1998 > allroad quattro without additional shift stage | Audi A6 1998 > allroad quattro without additional shift stage |
|  | Engine | 4.2 l - 250 kW - S6 | 2.7 l - 184 kW- Biturbo | 2.5 l - 132 kW - TDI |
| Ratio | Final drive | 37 : 9 = 4.111 | 35 : 8 = 4.375 | 35 : 8 = 4.375 |
| Z2 :Z1=i | 1st gear | 28 : 8 = 3.500 | 30 : 8 = 3.750 | 30 : 8 = 3.750 |
|  | 2nd gear | 34 : 18 = 1.889 | 35 : 17 = 2.059 | 35 : 17 = 2.059 |
|  | 3rd gear | 33 : 25 = 1.320 | 34 : 24 = 1.417 | 33 : 25 = 1.320 |
|  | 4th gear | 30 : 29 = 1.034 | 30 : 28 = 1.071 | 28 : 30 = 0.933 |
|  | 5th gear | 30 : 35 = 0.857 | 30 : 35 = 0.857 | 27 : 37 = 0.730 |
|  | 6th gear | 27 : 37 = 0.730 | 27 : 37 = 0.730 | 24 : 40 = 0.600 |
|  | Reverse gear | 38 : 11 = 3.455 | 38 : 11 = 3.455 | 38 : 11 = 3.455 |

| Code letters |  | EEY | EHS | EHT |
| --- | --- | --- | --- | --- |
| Speedometer |  | electronic |  |  |
| Capacity |  | 3.4 litres 1)2) | 3.4 litres 1)2) | 3.4 litres 1)2) |
| Specification |  | Gear oil G 052 911 A SAE 75 W 90 (synthetic oil) |  |  |
| Clutch mechanism |  | hydraulic |  |  |
| Clutch plate ø |  | 240 mm3) |  |  |
| Drive shaft flange ø |  | 108 mm | 88 mm5) | 88 mm5) |
| Overall ratio iov. in top gear |  | 3.000 | 3.193 | 2.625 |
| Assignment of rear final drive Code letters |  | DSA EUU | ENX | ENX |

1)    Oil capacity was reduced

2)    Oil capacity includes 0.3 l for gearbox oil cooler and oil pipe. When replacing gearbox, oil cooler and oil pipes have to be completely drained

3)    with dual mass flywheel

5)    for tripod joint drive shafts with 88 mm centring diameter

| Manual gearbox |  | 6-speed 01E, four-wheel drive |  |  |
| --- | --- | --- | --- | --- |
| Code letters |  | EKM | ETH | ETJ |
| Production | from | 08.99 | 10.99 | 10.99 |
|  | to |  |  |  |
| Allocation | Model | Audi A6 1998 > | Audi A6 1998 > allroad quattro with additional shift stage | Audi A6 1998 > allroad quattro with additional shift stage |
|  | Engine | 2.5 l - 132 kW - TDI | 2.5 l - 132 kW - TDI | 2.7 l - 184 kW- Biturbo |
| Ratio | Additional shift stage | - | i = 1.544 | i = 1.544 |
| Ratio | Final drive | 35 : 9 = 3.889 | 35 : 8 = 4.375 | 35 : 8 = 4.375 |
| Z2 :Z1=i | 1st gear | 30 : 8 = 3.750 | 30 : 8 = 3.750 | 30 : 8 = 3.750 |
|  | 2nd gear | 35 : 17 = 2.059 | 35 : 17 = 2.059 | 35 : 17 = 2.059 |
|  | 3rd gear | 33 : 25 = 1.320 | 33 : 25 = 1.320 | 34 : 24 = 1.417 |
|  | 4th gear | 28 : 30 = 0.933 | 28 : 30 = 0.933 | 30 : 28 = 1.071 |
|  | 5th gear | 27 : 38 = 0.711 | 27 : 37 = 0.730 | 30 : 35 = 0.857 |
|  | 6th gear | 23 : 41 = 0.561 | 24 : 40 = 0.600 | 27 : 37 = 0.730 |
|  | Reverse gear | 38 : 11 = 3.455 | 38 : 11 = 3.455 | 38 : 11 = 3.455 |

| Code letters |  | EKM | ETH | ETJ |
| --- | --- | --- | --- | --- |
| Speedometer |  | electronic |  |  |
| Capacity |  | 3.4 litres 1)2) | 3.4 litres 1)2) | 3.4 litres 1)2) |
| Specification |  | Gear oil G 052 911 A SAE 75 W 90 (synthetic oil) |  |  |
| Clutch mechanism |  | hydraulic |  |  |
| Clutch plate ø |  | 240 mm3) |  |  |
| Drive shaft flange ø |  | 130 mm4) | 88 mm5) | 88 mm5) |
| Overall ratio iov. in top gear |  | 2.182 | 2.625 | 3.193 |
| Assignment of rear final drive Code letters |  | EUT | ENX | ENX |

1)    Oil capacity was reduced

2)    Oil capacity includes 0.3 l for gearbox oil cooler and oil pipe. When replacing gearbox, oil cooler and oil pipes have to be completely drained

3)    with dual mass flywheel

4)    for tripod joint drive shafts with 130 mm centring diameter

5)    for tripod joint drive shafts with 88 mm centring diameter

| Manual gearbox |  | 6-speed 01E, four-wheel drive |  |  |
| --- | --- | --- | --- | --- |
| Code letters |  | FGY | FGZ | FPW |
| Production | from | 10.99 | 10.99 | 03.01 |
|  | to |  |  |  |
| Allocation | Model | Audi A6 1998 > allroad quattro with additional shift stage | Audi A6 1998 > allroad quattro with additional shift stage | Audi A6 1998 > |
|  | Engine | 2.7 l - 184 kW- Biturbo | 2.5 l - 132 kW - TDI | 3.0 l -162 kW |
| Ratio | Additional shift stage | i = 1.544 | i = 1.544 | - |
| Ratio | Final drive | 35 : 8 = 4.375 | 35 : 8 = 4.375 | 35 : 8 = 4.375 |
| Z2 :Z1=i | 1st gear | 30 : 8 = 3.750 | 30 : 8 = 3.750 | 28 : 8 = 3.500 |
|  | 2nd gear | 35 : 17 = 2.059 | 35 : 17 = 2.059 | 34 : 18 = 1.889 |
|  | 3rd gear | 34 : 24 = 1.417 | 33 : 25 = 1.320 | 33 : 25 = 1.320 |
|  | 4th gear | 30 : 28 = 1.071 | 28 : 30 = 0.933 | 30 : 29 = 1.034 |
|  | 5th gear | 30 : 35 = 0.857 | 27 : 37 = 0.730 | 30 : 35 = 0.857 |
|  | 6th gear | 27 : 37 = 0.730 | 24 : 40 = 0.600 | 27 : 37 = 0.730 |
|  | Reverse gear | 38 : 11 = 3.455 | 38 : 11 = 3.455 | 38 : 11 = 3.455 |

| Code letters |  | FGY | FGZ | FPW |
| --- | --- | --- | --- | --- |
| Speedometer |  | electronic |  |  |
| Capacity |  | 3.4 litres 1)2) | 3.4 litres 1)2) | 2.5 litres 1) |
| Specification |  | Gear oil G 052 911 A SAE 75 W 90 (synthetic oil) |  |  |
| Clutch mechanism |  | hydraulic |  |  |
| Clutch plate ø |  | 240 mm3) |  |  |
| Drive shaft flange ø |  | 88 mm5) | 88 mm5) | 130 mm4) |
| Overall ratio iov. in top gear |  | 3.193 | 2.625 | 3.193 |
| Assignment of rear final drive Code letters |  | ENX | ENX | EXY |

1)    Oil capacity was reduced

2)    Oil capacity includes 0.3 l for gearbox oil cooler and oil pipe. When replacing gearbox, oil cooler and oil pipes have to be completely drained

3)    with dual mass flywheel

4)    for tripod joint drive shafts with 130 mm centring diameter

5)    for tripod joint drive shafts with 88 mm centring diameter

| Manual gearbox |  | 6-speed 01E, four-wheel drive |  |  |
| --- | --- | --- | --- | --- |
| Code letters |  | FRP | FTF | FTG |
| Production | from | 05.01 | 05.01 | 05.01 |
|  | to |  |  |  |
| Allocation | Model | Audi A6 1998 > | Audi A6 1998 > | Audi A6 1998 > |
|  | Engine | 4.2 l - 250 kW - S6 | 2.5 l - 110 kW - TDI | 2.7 l - 169 kW- Biturbo |
| Ratio | Additional shift stage | - | - | - |
| Ratio | Final drive | 37 : 9 = 4.111 | 37 : 9 = 4.111 | 37 : 9 = 4.111 |
| Z2 :Z1=i | 1st gear | 28 : 8 = 3.500 | 28 : 8 = 3.500 | 28 : 8 = 3.500 |
|  | 2nd gear | 34 : 18 = 1.889 | 34 : 18 = 1.889 | 34 : 18 = 1.889 |
|  | 3rd gear | 33 : 25 = 1.320 | 32 : 26 = 1.231 | 32 : 26 = 1.231 |
|  | 4th gear | 30 : 29 = 1.034 | 27 : 31 = 0.871 | 29 : 30 = 0.967 |
|  | 5th gear | 30 : 35 = 0.857 | 26 : 39 = 0.667 | 29 : 36 = 0.806 |
|  | 6th gear | 27 : 37 = 0.730 | 23 : 41 = 0.561 | 26 : 38 = 0.684 |
|  | Reverse gear | 38 : 11 = 3.455 | 38 : 11 = 3.455 | 38 : 11 = 3.455 |

| Code letters |  | FRP | FTF | FTG |
| --- | --- | --- | --- | --- |
| Speedometer |  | electronic |  |  |
| Capacity |  | 3.4 litres 1)2) | 2.5 litres 1) | 2.5 litres 1) |
| Specification |  | Gear oil G 052 911 A SAE 75 W 90 (synthetic oil) |  |  |
| Clutch mechanism |  | hydraulic |  |  |
| Clutch plate ø |  | 240 mm3) |  |  |
| Drive shaft flange ø |  | 108 mm | 130 mm4) | 108 mm |
| Overall ratio iov. in top gear |  | 3.000 | 2.306 | 2.813 |
| Assignment of rear final drive Code letters |  | EUU | EUU | EUU |

1)    Oil capacity was reduced

2)    Oil capacity includes 0.3 l for gearbox oil cooler and oil pipe. When replacing gearbox, oil cooler and oil pipes have to be completely drained

3)    with dual mass flywheel

4)    for tripod joint drive shafts with 130 mm centring diameter

| Manual gearbox |  | 6-speed 01E, four-wheel drive |  |  |
| --- | --- | --- | --- | --- |
| Code letters |  | FTH | FTJ | FTL |
| Production | from | 05.01 | 05.01 | 05.01 |
|  | to |  |  |  |
| Allocation | Model | Audi A6 1998 > allroad quattro without additional shift stage | Audi A6 1998 > allroad quattro without additional shift stage | Audi A6 1998 > |
|  | Engine | 2.5 l - 132 kW - TDI | 2.7 l - 184 kW- Biturbo | 2.5 l - 132 kW - TDI |
| Ratio | Additional shift stage | - | - | - |
| Ratio | Final drive | 35 : 8 = 4.375 | 35 : 8 = 4.375 | 35 : 9 = 3.889 |
| Z2 :Z1=i | 1st gear | 30 : 8 = 3.750 | 30 : 8 = 3.750 | 30 : 8 = 3.750 |
|  | 2nd gear | 35 : 17 = 2.059 | 35 : 17 = 2.059 | 35 : 17 = 2.059 |
|  | 3rd gear | 33 : 25 = 1.320 | 34 : 24 = 1.417 | 33 : 25 = 1.320 |
|  | 4th gear | 28 : 30 = 0.933 | 30 : 28 = 1.071 | 28 : 30 = 0.933 |
|  | 5th gear | 27 : 37 = 0.730 | 30 : 35 = 0.857 | 27 : 38 = 0.711 |
|  | 6th gear | 24 : 40 = 0.600 | 27 : 37 = 0.730 | 23 : 41 = 0.561 |
|  | Reverse gear | 38 : 11 = 3.455 | 38 : 11 = 3.455 | 38 : 11 = 3.455 |

| Code letters |  | FTH | FTJ | FTL |
| --- | --- | --- | --- | --- |
| Speedometer |  | electronic |  |  |
| Capacity |  | 3.4 litres 1)2) | 3.4 litres 1)2) | 3.4 litres 1)2) |
| Specification |  | Gear oil G 052 911 A SAE 75 W 90 (synthetic oil) |  |  |
| Clutch mechanism |  | hydraulic |  |  |
| Clutch plate ø |  | 240 mm3) |  |  |
| Drive shaft flange ø |  | 88 mm5) | 88 mm5) | 130 mm4) |
| Overall ratio iov. in top gear |  | 2.625 | 3.193 | 2.182 |
| Assignment of rear final drive Code letters |  | ENX | ENX | EUT |

1)    Oil capacity was reduced

2)    Oil capacity includes 0.3 l for gearbox oil cooler and oil pipe. When replacing gearbox, oil cooler and oil pipes have to be completely drained

3)    with dual mass flywheel

4)    for tripod joint drive shafts with 130 mm centring diameter

5)    for tripod joint drive shafts with 88 mm centring diameter

| Manual gearbox |  | 6-speed 01E, four-wheel drive |  |  |
| --- | --- | --- | --- | --- |
| Code letters |  | FTN | FTP | FTQ |
| Production | from | 05.01 | 05.01 | 05.01 |
|  | to |  |  |  |
| Allocation | Model | Audi A6 1998 > allroad quattro with additional shift stage | Audi A6 1998 > allroad quattro with additional shift stage | Audi A6 1998 > allroad quattro with additional shift stage |
|  | Engine | 2.5 l - 132 kW - TDI | 2.5 l - 132 kW - TDI | 2.7 l - 184 kW- Biturbo |
| Ratio | Additional shift stage | i = 1.544 | i = 1.544 | i = 1.544 |
| Ratio | Final drive | 35 : 8 = 4.375 | 35 : 8 = 4.375 | 35 : 8 = 4.375 |
| Z2 :Z1=i | 1st gear | 30 : 8 = 3.750 | 30 : 8 = 3.750 | 30 : 8 = 3.750 |
|  | 2nd gear | 35 : 17 = 2.059 | 35 : 17 = 2.059 | 35 : 17 = 2.059 |
|  | 3rd gear | 33 : 25 = 1.320 | 33 : 25 = 1.320 | 34 : 24 = 1.417 |
|  | 4th gear | 28 : 30 = 0.933 | 28 : 30 = 0.933 | 30 : 28 = 1.071 |
|  | 5th gear | 27 : 37 = 0.730 | 27 : 37 = 0.730 | 30 : 35 = 0.857 |
|  | 6th gear | 24 : 40 = 0.600 | 24 : 40 = 0.600 | 27 : 37 = 0.730 |
|  | Reverse gear | 38 : 11 = 3.455 | 38 : 11 = 3.455 | 38 : 11 = 3.455 |

| Code letters |  | FTN | FTP | FTQ |
| --- | --- | --- | --- | --- |
| Speedometer |  | electronic |  |  |
| Capacity |  | 3.4 litres 1)2) | 3.4 litres 1)2) | 3.4 litres 1)2) |
| Specification |  | Gear oil G 052 911 A SAE 75 W 90 (synthetic oil) |  |  |
| Clutch mechanism |  | hydraulic |  |  |
| Clutch plate ø |  | 240 mm3) |  |  |
| Drive shaft flange ø |  | 88 mm5) | 88 mm5) | 88 mm5) |
| Overall ratio iov. in top gear |  | 2.625 | 2.625 | 3.193 |
| Assignment of rear final drive Code letters |  | ENX | ENX | ENX |

1)    Oil capacity was reduced

2)    Oil capacity includes 0.3 l for gearbox oil cooler and oil pipe. When replacing gearbox, oil cooler and oil pipes have to be completely drained

3)    with dual mass flywheel

5)    for tripod joint drive shafts with 88 mm centring diameter

| Manual gearbox |  | 6-speed 01E, four-wheel drive |  |  |
| --- | --- | --- | --- | --- |
| Code letters |  | FTR |  |  |
| Production | from | 05.01 |  |  |
|  | to |  |  |  |
| Allocation | Model | Audi A6 1998 > allroad quattro with additional shift stage |  |  |
|  | Engine | 2.7 l - 184 kW- Biturbo |  |  |
| Ratio | Additional shift stage | i = 1.544 |  |  |
| Ratio | Final drive | 35 : 8 = 4.375 |  |  |
| Z2 :Z1=i | 1st gear | 30 : 8 = 3.750 |  |  |
|  | 2nd gear | 35 : 17 = 2.059 |  |  |
|  | 3rd gear | 34 : 24 = 1.417 |  |  |
|  | 4th gear | 30 : 28 = 1.071 |  |  |
|  | 5th gear | 30 : 35 = 0.857 |  |  |
|  | 6th gear | 27 : 37 = 0.730 |  |  |
|  | Reverse gear | 38 : 11 = 3.455 |  |  |

| Code letters |  | FTR |  |  |
| --- | --- | --- | --- | --- |
| Speedometer |  | electronic |  |  |
| Capacity |  | 3.4 litres 1)2) |  |  |
| Specification |  | Gear oil G 052 911 A SAE 75 W 90 (synthetic oil) |  |  |
| Clutch mechanism |  | hydraulic |  |  |
| Clutch plate ø |  | 240 mm3) |  |  |
| Drive shaft flange ø |  | 88 mm5) |  |  |
| Overall ratio iov. in top gear |  | 3.193 |  |  |
| Assignment of rear final drive Code letters |  | ENX |  |  |

1)    Oil capacity was reduced

2)    Oil capacity includes 0.3 l for gearbox oil cooler and oil pipe. When replacing gearbox, oil cooler and oil pipes have to be completely drained

3)    with dual mass flywheel

5)    for tripod joint drive shafts with 88 mm centring diameter