| Renewing oil retention valves. Note In the event of irregular valve noise which disappears after a lengthy drive but repeatedly re-occurs when travelling short distances, the oil retention valves must be renewed.. Removing. –; Remove intake manifold → Rep. Gr.24.. |  |  |
| --- | --- | --- |

| –; Remove crankcase breather pipes -1- and -2-.. –; Unbolt cover -3- below intake manifold.. |  |  |
| --- | --- | --- |

| –; Unscrew oil retention valves -1- and -2-.. Installing. Installation is carried out in the reverse order; note the following:. Note ; Renew gaskets, seals and O-rings.. ; Secure all hose connections with the correct type of hose clips (same as original equipment) → Parts catalogue.. –; Install intake manifold → Rep. Gr.24.. Tightening torque. |  |  |
| --- | --- | --- |

| Component |  | Nm |
| --- | --- | --- |
| Oil retention valve to cylinder block |  | 20 |
| Cover to cylinder block |  | 10 |