Thursday,06,March,2014,21:20:03:10047

VCDS -- Windows Based VAG/VAS Emulator

VCDS Version: 12.12.2.0 (x64)

Data version: 20140212

VIN: WAUZZZ4E64N015552   License Plate:

Mileage: 126280km-78466mi   Repair Order:

--------------------------------------------------------------------------------

--------------------------------------------------------------------------------

Chassis Type: 4E (4E0)

Scan: 01 02 03 05 06 07 08 09 15 16 17 19 1E 25 34 36 37 38 42 46

47 4F 52 53 56 57 61 62 72 76 77

VIN: WAUZZZ4E64N015552   Mileage: 126280km/78466miles

00-Steering Angle Sensor -- Status: OK 0000

01-Engine -- Status: OK 0000

02-Auto Trans -- Status: OK 0000

03-ABS Brakes -- Status: OK 0000

05-Acc/Start Auth. -- Status: OK 0000

06-Seat Mem. Pass -- Status: OK 0000

07-Control Head -- Status: Malfunction 0010

08-Auto HVAC -- Status: Malfunction 0010

09-Cent. Elect. -- Status: OK 0000

15-Airbags -- Status: OK 0000

16-Steering wheel -- Status: OK 0000

17-Instruments -- Status: OK 0000

19-CAN Gateway -- Status: OK 0000

1E-Media Player 2 -- Status: OK 0000

25-Immobilizer -- Status: OK 0000

34-Level Control -- Status: Malfunction 0010

36-Seat Mem. Drvr -- Status: OK 0000

37-Navigation -- Status: OK 0000

38-Roof Electronics -- Status: OK 0000

42-Door Elect, Driver -- Status: OK 0000

46-Central Conv. -- Status: Malfunction 0010

47-Sound System -- Status: OK 0000

4F-Centr. Electr. II -- Status: OK 0000

52-Door Elect, Pass. -- Status: OK 0000

53-Parking Brake -- Status: Malfunction 0010

56-Radio -- Status: OK 0000

57-TV Tuner -- Status: OK 0000

61-Battery Regul. -- Status: Malfunction 0010

62-Door, Rear Left -- Status: OK 0000

72-Door, Rear Right -- Status: OK 0000

76-Park Assist -- Status: OK 0000

77-Telephone -- Status: OK 0000

-------------------------------------------------------------------------------

Address 01: Engine        Labels: 077-910-560-BFM.lbl

Part No SW: 4E0 910 560 Q    HW: 4E0 907 560

Component: 4.2L V8/5V      G   0030

Revision: 00000000    Serial number: AUX3Z0CN128606

Coding: 0011773

Shop #: WSC 00759 210 91870

VCID: 2F6D17B7BE3A736632D-807A

No fault code found.

Readiness: 0000 0000

-------------------------------------------------------------------------------

Address 02: Auto Trans        Labels: 09E-927-156.lbl

Part No SW: 4E0 910 156 E    HW: 09E 927 156 A

Component: AG6 09E 4,2L5V RdW  0110

Revision: 00000001    Serial number:        0025530

Coding: 0000001

Shop #: WSC 02313 785 00200

VCID: 2447E89BE7FCC03EDFF-8071

No fault code found.

-------------------------------------------------------------------------------

Address 03: ABS Brakes        Labels: 4E0-910-517.lbl

Part No SW: 4E0 910 517 B    HW: 4E0 614 517 M

Component: ESP 5.7 allrad  H44 0500

Coding: 0007597

Shop #: WSC 02313 000 00000

VCID: 224BEE83FDE8CE0EAD3-8077

No fault code found.

-------------------------------------------------------------------------------

Address 05: Acc/Start Auth.        Labels: 4E0-910-131.lbl

Part No SW: 4E0 910 131     HW: 4E0 909 131

Component:     FBS AUDID3  H03 0130

Revision: 01303703    Serial number: AUX3Z0CN128606

Coding: 0264192

Shop #: WSC 02311 785 00200

VCID: D8BF0C6B6B34D4DE037-808D

Subsystem 1 - Part No: XXXXXXXXXXX

Component: ELV                 XXXX

No fault code found.

-------------------------------------------------------------------------------

Address 06: Seat Mem. Pass        Labels: None

Part No SW: 4E0 910 760 C    HW: 4E0 959 760

Component: Sitzmemory BFS H07  0060

Revision: 00000000    Serial number: 5DS 008 795-03

Coding: 3670624

Shop #: WSC 00000 785 00200

VCID: 2345EB87E2F2C706A65-8076

No fault code found.

-------------------------------------------------------------------------------

Address 07: Control Head        Labels: 4E0-910-7xx-07.lbl

Part No SW: 4E0 910 732 L    HW: 4E0 035 729 A

Component: Interfacebox    H43 3460

Revision: 00000000    Serial number: 300KL037527650

Coding: 0000002

Shop #: WSC 00759 210 91870

VCID: 2B5503A78A220F461E5-807E

Subsystem 1 - Part No: 4E0 910 609 A

Component: Bedienteil MMI  H42 0020

Subsystem 2 - Serial number: 322KF037506040

Subsystem 3 - Serial number: 7425.11.030000000000010113ÿ

Subsystem 4 - Serial number: 010113ÿ

1 Fault Found:

00658 - Control Head

004 - No Signal/Communication - Intermittent

Freeze Frame:

Fault Status: 00100100

Fault Priority: 5

Fault Frequency: 2

Reset counter: 40

Mileage: 126285 km

Time Indication: 0

Date: 2014.03.06

Time: 19:32:27

-------------------------------------------------------------------------------

Address 08: Auto HVAC        Labels: None

Part No SW: 4E0 910 043     HW: 4E0 820 043 A

Component: KLIMAAUTOMAT    016 0240

Revision: 00000022    Serial number: 00000000030929

Coding: 1101889

Shop #: WSC 02311 785 00200

VCID: DABB16631538C6CE353-808F

1 Fault Found:

01857 - Air Outlet Sensor; Middle Left (G347)

007 - Short to Ground - Intermittent

Freeze Frame:

Fault Status: 00100111

Fault Priority: 3

Fault Frequency: 37

Reset counter: 254

Mileage: 125531 km

Time Indication: 0

Date: 2014.02.04

Time: 07:55:36

-------------------------------------------------------------------------------

Address 09: Cent. Elect.        Labels: 4E0-910-279.lbl

Part No SW: 4E0 910 279 E    HW: 4E0 907 279 E

Component: ILM Fahrer      H11 0100

Coding: 0100001

Shop #: WSC 02313 785 00200

VCID: 2A5B06A385D8164EE53-807F

Subsystem 1 - Part No: 4E2 910 113

Component: Wischer WWS D3  H30 0320

Coding: 00000030

Shop #: WSC 02313

No fault code found.

-------------------------------------------------------------------------------

Address 15: Airbags        Labels: 4E0-910-655-84.lbl

Part No SW: 4E0 910 655     HW: 4E0 959 655 G

Component: Airbag 8.4E+    H10 0940

Coding: 0431707

Shop #: WSC 02313 785 00200

VCID: E3C52B8722720706665-80B6

No fault code found.

-------------------------------------------------------------------------------

Address 16: Steering wheel        Labels: 4E0-910-549.lbl

Part No SW: 4E0 910 549     HW: 4E0 953 549

Component: Lenksäulenmodul H13 0040

Coding: 0112041

Shop #: WSC 02311 785 00200

VCID: E5C1359F2C467936881-80B0

Subsystem 1 - Part No: XXXXXXXXXXX

Component: Lenkradmodul    H03 0070

No fault code found.

-------------------------------------------------------------------------------

Address 17: Instruments        Labels: 4E0-910-xxx-17.lbl

Part No SW: 4E0 910 900     HW: 4E0 920 950 C

Component: KOMBIINSTR. VDO H44 0080

Revision: D13         Serial number: AUDID3CN001526

Coding: 0000403

Shop #: WSC 00568 210 80708

VCID: DCB7107B1F0C38FE27F-8089

No fault code found.

-------------------------------------------------------------------------------

Address 19: CAN Gateway        Labels: 4E0-910-468.lbl

Part No SW: 4E0 910 468     HW: 4E0 907 468 A

Component: Gateway         H07 0070

Revision: 07          Serial number: 1400303A062527

Coding: FCFE5BDC2B3504

Shop #: WSC 02313 785 00200

VCID: E5C1359F2C467936881-80B0

No fault code found.

-------------------------------------------------------------------------------

Address 1E: Media Player 2        Labels: 4E0-910-111.lbl

Part No SW: 4E0 910 111 F    HW: 4E0 035 111

Component: CD-Changer      H43 0470

Revision: 00000501    Serial number: AUZ5ZBC1026902

Shop #: WSC 00000 000 00000

VCID: 1C37D07BDF8CF8FE67F-8049

No fault code found.

-------------------------------------------------------------------------------

Address 25: Immobilizer        Labels: 4E0-910-131.lbl

Part No SW: 4E0 910 131     HW: 4E0 909 131

Component:     FBS AUDID3  H03 0130

Revision: 01303703    Serial number: AUX3Z0CN128606

Coding: 0264192

Shop #: WSC 02311 785 00200

VCID: D8BF0C6B6B34D4DE037-808D

Subsystem 1 - Part No: XXXXXXXXXXX

Component: ELV                 XXXX

XXXXXXXXXXX ELV                 XXXX

No fault code found.

-------------------------------------------------------------------------------

Address 34: Level Control        Labels: 4E0-910-553.lbl

Part No SW: 4E0 910 553 E    HW: 4E0 907 553 E

Component: LUFTFDR.-CDC    H05 4290

Coding: 0015510

Shop #: WSC 02154 210 01865

VCID: 2541F59FECC63936C81-8070

1 Fault Found:

01487 - System Function Test

008 - Implausible Signal

Freeze Frame:

Fault Status: 11101000

Fault Priority: 2

Fault Frequency: 1

Reset counter: 225

Mileage: 126285 km

Time Indication: 0

Date: 2014.03.06

Time: 20:42:11

-------------------------------------------------------------------------------

Address 36: Seat Mem. Drvr        Labels: None

Part No SW: 4E0 910 760 C    HW: 4E0 959 760

Component: Sitzmemory FS  H07  0060

Revision: 00000000    Serial number: 5DS 008 795-03

Coding: 3670626

Shop #: WSC 00000 785 00200

VCID: 2345EB87E2F2C706A65-8076

No fault code found.

-------------------------------------------------------------------------------

Address 37: Navigation        Labels: 4E0-910-88x-37.lbl

Part No SW: 4E0 910 887 D    HW: 4E0 919 887 C

Component: MNS ECE         H42 0550

Revision: 00000000    Serial number: 305K-037549593

Shop #: WSC 02313 785 00200

VCID: 2E530AB3B9307A6E39B-807B

No fault code found.

-------------------------------------------------------------------------------

Address 38: Roof Electronics        Labels: 4E0-910-135.lbl

Part No SW: 4E0 910 135 A    HW: 4E0 907 135 A

Component: Dachknoten D3   H01 0090

Serial number: 00000332100001

Coding: 0036000

Shop #: WSC 02313 785 00200

VCID: 1D31DD7FC496F1F6901-8048

No fault code found.

-------------------------------------------------------------------------------

Address 42: Door Elect, Driver        Labels: 4E1-910-802-42.lbl

Part No SW: 4E1 910 802     HW: 4E2 959 802

Component: Türsteuer.FS BRM005 0110

Coding: 0027054

Shop #: WSC 02311 785 00200

VCID: DEB31A7309102AEE49B-808B

No fault code found.

-------------------------------------------------------------------------------

Address 46: Central Conv.        Labels: 4E0-910-289.lbl

Part No SW: 4E0 910 289 A    HW: 4E0 907 289

Component: Komfortgeraet   H03 0030

Revision: 00300003    Serial number:  W040311190249

Coding: 0004825

Shop #: WSC 02313 785 00200

VCID: 275DFF9796CA2B26FAD-8072

Subsystem 1 - Part No: 4E0 907 719

Component: Neigungssensor  H02 0020

2 Faults Found:

01134 - Alarm Horn (H12)

012 - Electrical Fault in Circuit

Freeze Frame:

Fault Status: 01101100

Fault Priority: 3

Fault Frequency: 254

Reset counter: 18

Mileage: 50522 km

Time Indication: 0

Date: 2010.04.29

Time: 18:47:08

01134 - Alarm Horn (H12)

004 - No Signal/Communication

Freeze Frame:

Fault Status: 01101100

Fault Priority: 3

Fault Frequency: 254

Reset counter: 18

Mileage: 50522 km

Time Indication: 0

Date: 2010.04.29

Time: 18:47:08

-------------------------------------------------------------------------------

Address 47: Sound System        Labels: 4E0-910-223.lbl

Part No SW: 4E0 910 223 L    HW: 4E0 035 223

Component: Amp.-Basis      H42 0290

Revision: 00000000    Serial number: 00000037520028

Coding: 0000007

Shop #: WSC 02313 785 00200

VCID: 2643F29391C0322EC1B-8073

No fault code found.

-------------------------------------------------------------------------------

Address 4F: Centr. Electr. II        Labels: None

Part No SW: 4E0 910 280 A    HW: 4E0 907 280 A

Component: ILM Beifahrer   H08 0020

Revision:             Serial number: 00000000028142

Coding: 0010101

Shop #: WSC 02313 785 00200

VCID: 1E33DA73C990EAEE89B-804B

No fault code found.

-------------------------------------------------------------------------------

Address 52: Door Elect, Pass.        Labels: 4E1-910-802-52.lbl

Part No SW: 4E1 910 802     HW: 4E2 959 801

Component: Türsteuer.BF BRM005 0110

Coding: 0027054

Shop #: WSC 02311 785 00200

VCID: DEB31A7309102AEE49B-808B

No fault code found.

-------------------------------------------------------------------------------

Address 53: Parking Brake        Labels: 4E0-910-801.clb

Part No SW: 4E0 910 801 A    HW: 4E0 907 801

Component: EPB  AV1C1010   H01 1010

Revision: AV1C1010

Coding: 0000531

Shop #: WSC 54064 210 66681

VCID: 1D31DD7FC496F1F6901-8048

4 Faults Found:

02443 - Control Head

012 - Electrical Fault in Circuit - MIL ON

Freeze Frame:

Fault Status: 11101100

Fault Priority: 2

Fault Frequency: 62

Reset counter: 225

Mileage: 126008 km

Time Indication: 0

Date: 2014.02.26

Time: 18:57:26

Freeze Frame:

Voltage: 13.15 V

Count: 40

02442 - Function Lamp in Control Head

009 - Open or Short to Ground

Freeze Frame:

Fault Status: 01101001

Fault Priority: 6

Fault Frequency: 2

Reset counter: 225

Mileage: 126280 km

Time Indication: 0

Date: 2014.03.06

Time: 19:35:01

Freeze Frame:

Voltage: 10.66 V

Count: 65

02428 - Left Parking Brake Motor (V282) Clamping Force Not Reached

000 - - - Intermittent

Freeze Frame:

Fault Status: 00110000

Fault Priority: 4

Fault Frequency: 1

Reset counter: 9

Mileage: 126280 km

Time Indication: 0

Date: 2014.03.06

Time: 19:33:42

Freeze Frame:

Voltage: 11.28 V

Count: 14

02596 - Rear Brake Pads Worn

008 - Implausible Signal

Freeze Frame:

Fault Status: 01111000

Fault Priority: 6

Fault Frequency: 1

Reset counter: 225

Mileage: 126280 km

Time Indication: 0

Date: 2014.03.06

Time: 19:33:42

Freeze Frame:

Voltage: 11.28 V

Count: 61

-------------------------------------------------------------------------------

Address 56: Radio        Labels: 4E0-910-541.lbl

Part No SW: 4E0 910 541 Q    HW: 4E0 035 541

Component: TUNER EU/US/RDW H41 0620

Revision: 00000000    Serial number: 304KK037553151

Coding: 0000001

Shop #: WSC 02313 785 00200

VCID: 2E530AB3B9307A6E39B-807B

No fault code found.

-------------------------------------------------------------------------------

Address 57: TV Tuner        Labels: 4E0-910-146.clb

Part No SW: 4E0 910 146 F    HW: 4E0 919 146

Component: TV analog TUNER H12 0290

Revision: 00000017    Serial number: 00000000004553

Coding: 0001003

Shop #: WSC 00000 785 00200

VCID: 2447E89BE7FCC03EDFF-8071

No fault code found.

-------------------------------------------------------------------------------

Address 61: Battery Regul.        Labels: 4E0-910-181.lbl

Part No SW: 4E0 910 181 A    HW: 4E0 915 181

Component: ENERGIEMANAGER  H02 0230

Revision: 00000000    Serial number: 00000005308435

Shop #: WSC 00976 210 10705

VCID: 1E33DA73C990EAEE89B-804B

Subsystem 1 - Part No: 000 915 105 AK

Component: von VA0  1310170394

Coding: 303030393135313035414B56413031333130313730333934

Shop #: WSC 00000 128 00289

5 Faults Found:

02276 - Quiescent Current Stage 5

000 - - - Intermittent

Freeze Frame:

Fault Status: 00100000

Fault Priority: 7

Fault Frequency: 16

Reset counter: 255

Mileage: 120819 km

Time Indication: 0

Date: 2013.03.07

Time: 11:15:44

02273 - Quiescent Current Stage 2

000 - - - Intermittent

Freeze Frame:

Fault Status: 00100000

Fault Priority: 7

Fault Frequency: 26

Reset counter: 255

Mileage: 120819 km

Time Indication: 0

Date: 2013.03.07

Time: 11:24:30

02272 - Quiescent Current Stage 1

000 - - - Intermittent

Freeze Frame:

Fault Status: 00100000

Fault Priority: 7

Fault Frequency: 108

Reset counter: 255

Mileage: 120821 km

Time Indication: 0

Date: 2013.03.07

Time: 15:14:05

02277 - Quiescent Current Stage 6

000 - - - Intermittent

Freeze Frame:

Fault Status: 00100000

Fault Priority: 7

Fault Frequency: 220

Reset counter: 255

Mileage: 123566 km

Time Indication: 0

Date: 2013.11.20

Time: 20:55:04

02274 - Quiescent Current Stage 3

000 - - - Intermittent

Freeze Frame:

Fault Status: 00100000

Fault Priority: 7

Fault Frequency: 223

Reset counter: 255

Mileage: 123573 km

Time Indication: 0

Date: 2013.11.21

Time: 09:15:35

-------------------------------------------------------------------------------

Address 62: Door, Rear Left        Labels: 4E0-910-802.lbl

Part No SW: 4E0 910 802 A    HW: 4E0 959 801 C

Component: Türsteuer.HL BRMH01 0040

Coding: 0027054

Shop #: WSC 02311 785 24776

VCID: 1E33DA73C990EAEE89B-804B

No fault code found.

-------------------------------------------------------------------------------

Address 72: Door, Rear Right        Labels: 4E0-910-802.lbl

Part No SW: 4E0 910 802 A    HW: 4E0 959 802 C

Component: Türsteuer.HR BRMH01 0040

Coding: 0027054

Shop #: WSC 02311 785 24776

VCID: 1E33DA73C990EAEE89B-804B

No fault code found.

-------------------------------------------------------------------------------

Address 76: Park Assist        Labels: 4E0-910-283.lbl

Part No SW: 4E0 910 283 A    HW: 4E0 919 283 B

Component: Parkhilfe 8-Kan H11 0010

Revision: --.--.--    Serial number: 00000333000126

Coding: 0001008

Shop #: WSC 02313 785 00200

VCID: 2149E18FF8EED516B49-8074

No fault code found.

-------------------------------------------------------------------------------

Address 77: Telephone        Labels: 4E0-910-333.lbl

Part No SW: 4E0 910 333 F    HW: 4E0 862 333 B

Component: TELEFONMODUL EU     0770

Revision: 00000001    Serial number: 00000000000000

Coding: 0000004

Shop #: WSC 00000 000 00000

VCID: 224BEE83FDE8CE0EAD3-8077

No fault code found.

End   ---------------------------------------------------------------------